LiDAR-Based Tree Detection in Mixed Forests

University of Luxembourg

Apoorv Dhyani

Paria Ighanian

## Exploratory Data Analysis (EDA)

### Dataset Overview

The dataset field_survey.geojson contains 3,602 rows and 11 columns, providing information about individual trees in the surveyed area. Each row represents a unique tree with attributes like species, diameter measurements, age, height, angle, and spatial coordinates.

### Species Distribution

Figure 1 illustrates the distribution of tree species in the surveyed area. Among the 9 distinct species, Spruce is the most common, making up 31.76% (1,144 trees) of the total. Other common species include Birch (18.77%, 676 trees), Fir (18.41%, 663 trees), and Aspen (16.60%, 598 trees). In contrast, the rarest species are Pine, Elm, and Willow, with Pine and Elm recorded as a single tree each, and Willow with 28 trees.

Figure 1 Tree Species Distribution in the Surveyed Area

These species can be classified into two groups: Coniferous (evergreens such as Spruce, Fir, and Pine) and Deciduous (leaf-shedding species like Birch, Aspen, and Tilia). Coniferous trees retain their needle-like leaves year-round, while Deciduous trees lose their broad leaves during specific seasons. In this classification, about half the dataset (50.2%) consists of Coniferous trees, with the other half being Deciduous.

### Diameter at Breast Height

Diameter at breast height (DBH) is a standard forestry measurement representing the tree's diameter at 1.37 meters above the ground. It is calculated as the mean of d1 and d2, which are perpendicular diameter measurements taken at breast height to account for irregular trunk shapes. Figure 2 shows a right-skewed distribution, with most tree diameters concentrated between 10 and 30 units and peaking around 15–20. This indicates a dominance of smaller-diameter trees, with fewer larger ones represented by the long tail.

Figure 2 Density plot of Diameter at Breast Height (DBH) measurements

Figure 3 shows the variation in DBH across the 9 tree species. Spruce has the largest variability, with a wide interquartile range (IQR) and a few outliers. In contrast, Willow and Alder have narrower distributions, indicating more uniform DBH values. Birch and Tilia have more outliers compared to the other species. Most species have a median DBH between 15 and 30 units. Pine and Elm, represented by only a single tree each, do not show any variability.

Figure 3 Distribution of Diameter at Breast Height (DBH) Across Tree Species

### Plot

This feature represents rectangular ground plots surveyed in the dataset. These plots are associated with both .tif and .las files. Figure 4 illustrates the distribution of tree species across these 10 plots. Each point represents an individual tree, color-coded by its species. The plots reveal varying species compositions, with some plots dominated by specific species. For instance, Plot 1 is largely populated with Aspens, while Plot 7 shows a predominance of Birch trees.

Figure 4 Species Distribution Across Rectangular Plots 1 to 10

Figure 5 provides a more detailed view of the species distribution within each individual plot.

Figure 5 Detailed View of Species Distribution Across Rectangular Plots 1 to 10

Figure 6 shows the RGB version of tree distribution across 10 rectangular plots, providing another perspective of the region.

Figure 6 RGB Images of Rectangular Plots 1 to 10

Moreover, Figure 7 shows the 3D version of point clouds in each plot, with the z-axis representing height.

Figure 7 3D Visualization of Point Clouds for Rectangular Plots 1 to 10

## Data Preprocessing

Through analyzing the loaded point clouds, we identified two primary classifications:

- Classification 2: Ground points

- Classification 5: High vegetation points

Figure 8 is a 3D visualization of one of the LiDAR plots, where ground points are shown in red for clarity. Additionally, 3D plots of other LiDAR point clouds have been saved in the folder: /docs/lidar_shifted_plots. Note that the coordinates in those visualizations were shifted solely for clearer display. Ground points are retained during preprocessing to normalize the height of all LiDAR points relative to the ground level. The height normalization is performed by interpolating a ground surface from points classified as ground (classification 2) and subtracting the ground height from the Z-values of all points. This ensures that heights are measured relative to the terrain, which is critical for accurately identifying trees. To handle outliers, the Z-value distributions of the LiDAR data are analyzed, as shown in Figure 9. The red dashed lines in the plots represent the upper and lower bounds determined using the interquartile range (IQR) method. Z-values outside these bounds are considered outliers and are removed during preprocessing to improve data quality. The preprocessed dataset is then split into 70% for training and 30% for testing.

Figure 8 LiDAR Point Cloud Visualization (Shifted)

Figure 9 Z Elevation Distribution for Vegetation Points Across LiDAR Plots

## Detection Method: Local Maxima Filtering Algorithm

The local_maxima_filter method identifies local maxima in a 3D point cloud by comparing the height of each point to its neighbors within a fixed radius. It begins by filtering out points below a specified height threshold. For each remaining point, it uses a KDTree (k-dimensional tree) to efficiently find neighboring points and determines the one with the highest elevation. If the current point is the highest within its neighborhood, it is classified as a local maximum and added to the results. The method also ensures that points and their neighbors are not processed multiple times. Ultimately, it returns a subset of points that are identified as local maxima. After identifying local maxima in the LiDAR data, the detected points are compared to ground truth tree locations derived from field surveys. This process involves several steps to ensure accurate evaluation:

- Loading Ground Truth Data:

For each plot, ground truth tree locations and their heights were extracted from the field survey dataset. The data was converted into a NumPy array for efficient processing, with tree locations in the X, Y coordinates and heights as Z-values.

- Filtering Candidates:

The detected local maxima were filtered to ensure they aligned with the ground truth area. This was achieved by cropping the detected candidates to the convex hull of the ground truth points, which removed extraneous detections outside the region of interest.

- Matching Detected Points with Ground Truth:

The matching process involved:

- Proximity Matching: Detected points were matched to ground truth trees if their horizontal distance was within 5 meters.

- Height Validation: To account for variability in tree heights, matches were further filtered to ensure the difference in Z-values (height) between detected and ground truth points did not exceed 3 meters.

- Matches were categorized as:

- True Positives (TP): Detected points correctly matched to ground truth trees.

- False Positives (FP): Detected points without corresponding ground truth matches.

- False Negatives (FN): Ground truth trees without corresponding detections.

The matching results were stored in a DataFrame for each plot, including the classification (TP, FP, FN), coordinates, tree heights, and distances. This made it easier to analyze and visualize the results.

### Evaluation

To assess the performance of the local maxima filtering method in detecting trees, the matching results were evaluated using common metrics:

Performance Metrics:

- Precision: The proportion of detected points that were correctly matched to ground truth trees (True Positives / (True Positives + False Positives)).

- Recall: The proportion of ground truth trees that were correctly detected (True Positives / (True Positives + False Negatives)).

- F1-Score: The harmonic mean of precision and recall, providing a balanced measure of detection performance.

- Average Distance: The average horizontal distance between matched ground truth and detected points, providing a measure of spatial accuracy.

Table 1 shows the performance metrics of the local maxima filtering (LMF) algorithm for different plots, with averages included in the last row to summarize overall performance.

| Plot | Recall | Precision | F1-Score | Avg. Distance |
| --- | --- | --- | --- | --- |
| 1 | 0.58 | 0.67 | 0.62 | 0.76 |
| 2 | 0.48 | 0.56 | 0.52 | 0.79 |
| 3 | 0.61 | 0.42 | 0.50 | 1.11 |
| 4 | 0.56 | 0.38 | 0.46 | 1.04 |
| 5 | 0.65 | 0.45 | 0.53 | 1.02 |
| 6 | 0.59 | 0.41 | 0.48 | 0.81 |
| 7 | 0.43 | 0.66 | 0.52 | 1.02 |
| 8 | 0.62 | 0.45 | 0.52 | 0.83 |
| 9 | 0.42 | 0.69 | 0.52 | 0.93 |
| 10 | 0.37 | 0.78 | 0.51 | 0.77 |
| Average | 0.53 | 0.55 | 0.52 | 0.91 |

Table 1 Performance Metrics of Local Maxima-Based Tree Detection Across Plots

Visualization of Detection Results:

The detection results of the local maxima filtering (LMF) algorithm were visualized for each plot to better understand its performance.

- Red Points: Detected trees (local maxima).

- Background Gradient: Elevation data from the LiDAR point cloud.

Figure 10 LMF-based tree detection results for 10 LiDAR plots

Figure 10 is saved in the results folder as lmf_tree_detections.png.

## Converting Point Cloud to Voxel Grid